Exposure Time Calculator and Performance Analysis

Written by :		R. Bacon

Institute : 		CRAL

Reference :		MUSE-MEM-SCI-051

Issue :			1.3

Date :			28/01/04

File :				muse_etc.doc

Distribution :		ESO

History:

- 1.0 – 22/12/03 – Initial version with inputs from Simon Morris and Ian Parry

- 1.1 – 09/01/04 – Comments by Richard Mc Dermid

- 1.2 – 15/01/04 – Comments by Luca Pasquini

- 1.3- 28/01/04 – Final Phase A release

1.	Introduction	3

2.	Documents	3

2.1.	Applicable documents	3

2.2.	Reference documents	3

3.	Acronyms	3

4.	Assumptions	5

5.	Results	6

5.1.	Wide-field mode	6

Extended source	6

Unresolved source	7

5.2.	Narrow field mode	8

Extended source	8

Unresolved source	8

6.	Analysis	9

6.1.	Wavelength dependency	9

6.2.	Improvement due to AO	9

6.3.	Noise regime	10

Wide field mode	10

Narrow field mode	10

6.4.	Centering star	10

6.5.	Other models of throughput	10

7.	Limitations	12

8.	Plan for phases B & C	12

ANNEX – ETC mathcad sheet	13

# Introduction

This document describes the MUSE exposure time calculator developed during phase A and discusses the instrument performances. Limiting flux and magnitude quoted in the Science Case document refer to these ETC results.

# Documents

## Applicable documents

| AD1 | MUSE Top Instrumental Parameters | MUSE-MEM-SCI-016 |
| --- | --- | --- |
| AD2 | MUSE AO analysis report | VLT-TRE-ESO-14675-2951 |
| AD3 | MUSE System analysis and budgets | MUSE-MEM-TEC-031 |
| AD4 | MUSE Instrument detector system specification | MUSE-MEM-TEC-024 |

## Reference documents

| RD1 | ESO ETC web pages |  |
| --- | --- | --- |
| RD2 | A flux calibrated, high resolution atlas of optical emission sky from UVES | Hanuschik R.W, 2003, A&A, 407, 1157 |
| RD4 | MUSE Science Case | MUSE-MEM-SCI-052 |

# Acronyms

| AD | Applicable Document |
| --- | --- |
| AO | Adaptive Optics |
| CCD | Charge-Coupled Device |
| ESO | European Southern Observatory |
| ETC | Exposure Time Calculator |
| FoV | Field of View |
| FWHM | Full Width Half Maximum |
| INM | Instrument Numerical Model |
| MUSE | Multi Unit Spectroscopic Explorer |
| NA | Not Applicable |
| NFM | Narrow Field Mode |
| PSF | Point Spread Function |
| R | Spectral Resolving Power |
| RD | Reference Document |
| S/N | Signal over noise |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| VLT | Very Large Telescope |
| WFM | Wide Field Mode |

# Assumptions

Mathematics and detail computation can be found in the appendix. All important assumptions and input parameters are shown in following table.

| Item | Assumptions | Remark |
| --- | --- | --- |
| Sky brightness | Continuum only (outside OH emission lines), reference RD2, moon aged of 5 days | S/N estimate is only valid outside OH lines (see RD1 for an estimate of the free OH fraction of spectral range) |
| Atmospheric extinction | Paranal extinction (reference RD1) | An airmass of 1 is assumed |
| Telescope effective area | 485425.1 cm² (reference RD1) |  |
| Instrument throughput in WF mode | Reference AD3 | Typical curve with adaptive secondary |
| Instrument throughput in HR mode | Provisionally taken equal to WF mode | Overestimated by 5% |
| CCD dark current | 3 electrons/hour (reference AD4) | Typical value |
| CCD readout | 4 electrons (reference AD4) | Fairchild 4k x 4k |
| Spatial PSF | 4 cases considered: poor (1.1 arcsec, 70%-tile) and good (0.65 arcsec, 30%-tile) seeing conditions, with and without AO. Simulations take into account PSF variation with wavelength (VLT-TRE-ESO-14675-2951). All PSFs are convolved with MUSE image quality, assumed to be 0.254 arcsec FWHM (AD1) | Current Paranal statistics (5/99-8/02) |
| Number of summed spatial pixels (1) | 3x3 pixels (0.6x0.6 arcsec²) in good seeing conditions and 4x4 pixels (0.8x0.8 arcsec²) in poor seeing conditions, both for AO and non AO observations | For spatially unresolved source. Object flux fraction recovered is 40-60%. |
| Spectral PSF | Gaussian shape of 2 pixels FWHM |  |
| Number of summed spectral pixels (2) | 3 pixels for emission line source, 1 or 10 pixels for respectively full and low spectral resolution continuum | 92% fraction of flux enclosed in case of emission line source |
| Exposure time | 1 hour | Limited by cosmic ray impacts |
| Number of exposure | 80 for deep field and 1 for shallow field |  |
| Limiting S/N | 5 for 1 resolution element | A resolution element being defined in (1) and (2) |

# Results

## Wide-field mode

### Extended source

The following two tables give the instrument performance at full spectral resolution in case of a spatially extended source with a flat continuum spectral distribution (AB surface magnitude) or an unresolved emission line (line flux by arcsec² in 10-19 erg.s-1.cm-2 units). Two cases are considered, a single exposure of 1 hour (shallow field) and a series of 80 exposures of 1 hour (deep field). Values are given at central wavelength of photometric band, except for B’ and z’ which are set to the limits of the MUSE spectral range.

Shallow Field Observations (1h) – Extended source

|  (µm) | Band | AB arcsec-2 | Line flux arcsec-2 |
| --- | --- | --- | --- |
| 0.465 | B’ | 20.5 | 595.6 |
| 0.55 | V | 21.2 | 221.3 |
| 0.64 | R | 21.3 | 154.1 |
| 0.79 | I | 20.8 | 153.4 |
| 0.93 | z’ | 20.0 | 221.9 |

Deep Field Observations (80x1h) – Extended source

|  (µm) | Band | AB arcsec-2 | Line flux arcsec-2 |
| --- | --- | --- | --- |
| 0.465 | B’ | 23.28 | 53.8 |
| 0.55 | V | 23.87 | 21.4 |
| 0.64 | R | 23.93 | 15.0 |
| 0.79 | I | 23.48 | 14.9 |
| 0.93 | z’ | 22.78 | 20.4 |

### Unresolved source

The next two tables display the limiting magnitude and line emission flux (in 10-19 erg.s-1.cm-2 units) in the case of spatially unresolved object. Various conditions have been explored for the spatial PSF: seeing limited observations (non AO column) and AO observation, each in two atmospheric conditions: good (seeing of 0.65 arcsec) and poor (seeing of 1.1 arcsec). To retrieve a significant fraction of the object flux, we have summed in an area of 0.6x0.6 arcsec² in the case of good seeing conditions and 0.8x0.8 arcsec² in the case of poor seeing. A low spectral resolution limiting magnitude at a tenth of the nominal spectral resolution is also given (obtained by summation of ten spectral pixels and assuming a flat continuum spectral distribution). Shallow (1h) and deep (80x1h) exposures are presented.

Shallow Field Observations (1h) – Unresolved Source

|  (µm) | Band |  | AB full R |  | AB R/10 |  | Line Flux |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Atm. cond | Poor | Good | Poor | Good | Poor | Good |
| 0.465 | B’ | Non AO | 21.7 | 22.1 | 23.0 | 23.5 | 223.3 | 144.4 |
|  |  | AO | 21.8 | 22.4 | 23.1 | 23.7 | 199.0 | 111.2 |
| 0.55 | V | Non AO | 22.4 | 22.8 | 23.7 | 24.2 | 84.2 | 53.5 |
|  |  | AO | 22.7 | 23.1 | 24.0 | 24.5 | 64.1 | 40.9 |
| 0.64 | R | Non AO | 22.5 | 22.9 | 23.8 | 24.3 | 56.4 | 36.3 |
|  |  | AO | 22.8 | 23.2 | 24.1 | 24.6 | 42.5 | 27.4 |
| 0.79 | I | Non AO | 22.1 | 22.6 | 23.4 | 23.9 | 52.2 | 33.4 |
|  |  | AO | 22.4 | 22.9 | 23.7 | 24.2 | 38.7 | 25.3 |
| 0.93 | z’ | Non AO | 21.4 | 21.9 | 22.8 | 23.2 | 68.7 | 44.8 |
|  |  | AO | 21.8 | 22.2 | 23.1 | 23.5 | 49.7 | 33.7 |

Deep Field Observations (80x1h) – Unresolved Source

|  (µm) | Band |  | AB full R |  | AB R/10 |  | Line Flux |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Atm. cond | Poor | Good | Poor | Good | Poor | Good |
| 0.465 | B’ | Non AO | 24.1 | 24.6 | 25.4 | 25.9 | 23.6 | 15.0 |
|  |  | AO | 24.3 | 24.9 | 25.5 | 26.2 | 21.1 | 11.6 |
| 0.55 | V | Non AO | 24.8 | 25.3 | 26.1 | 26.6 | 9.1 | 5.7 |
|  |  | AO | 25.1 | 25.6 | 26.4 | 26.9 | 6.9 | 4.4 |
| 0.64 | R | Non AO | 24.9 | 25.4 | 26.2 | 26.7 | 6.1 | 3.9 |
|  |  | AO | 25.2 | 25.7 | 26.5 | 27.0 | 4.6 | 2.9 |
| 0.79 | I | Non AO | 24.5 | 25.0 | 25.8 | 26.3 | 5.6 | 3.6 |
|  |  | AO | 24.9 | 25.3 | 26.1 | 26.6 | 4.2 | 2.7 |
| 0.93 | z’ | Non AO | 23.9 | 24.4 | 25.2 | 25.6 | 7.3 | 4.7 |
|  |  | AO | 24.3 | 24.7 | 25.5 | 26.0 | 5.3 | 3.5 |

## Narrow field mode

### Extended source

The following table gives the instrument performance at full spectral resolution in case of a spatially extended source with a flat continuum spectral distribution (AB surface magnitude) or an unresolved emission line (line flux by arcsec² in 10-19 erg.s-1.cm-2 units). Values are given at central wavelength of photometric band, except for B’ and z’ which are set to the limits of the MUSE spectral range.

Typical Observations (1h) – Extended source

|  (µm) | Band | AB arcsec-2 | Line flux arcsec-2 |
| --- | --- | --- | --- |
| 0.465 | B’ | 16.49 | 2564.0 |
| 0.55 | V | 17.21 | 702.6 |
| 0.64 | R | 17.35 | 472.6 |
| 0.79 | I | 16.95 | 480.2 |
| 0.93 | z’ | 15.93 | 899.3 |

### Unresolved source

The next tables display the limiting magnitude and line emission flux (in 10-19 erg.s-1.cm-2 units) in the case of spatially unresolved object.  Only AO observation in good seeing (0.65 arcsec) conditions is considered. To retrieve a significant fraction of the object flux, we have summed in an area of 0.075x0.075 arcsec. A low spectral resolution limiting magnitude at a tenth of the nominal spectral resolution is also given (obtained by summation of ten spectral pixels and assuming a flat continuum spectral distribution).

Typical Observations (1h) – Unresolved source

|  (µm) | Band | AB full R | AB R/10 | Line flux |
| --- | --- | --- | --- | --- |
| 0.465 | B’ | 21.08 | 23.06 | 214.8 |
| 0.55 | V | 22.00 | 24.49 | 42.6 |
| 0.64 | R | 22.30 | 24.85 | 22.8 |
| 0.79 | I | 22.09 | 24.61 | 18.6 |
| 0.93 | z’ | 21.66 | 23.75 | 28.6 |

# Analysis

Figure 1: Variation of throughput (dashed line) and limiting flux (solid line) with wavelength.

Wavelength dependency

The computed limiting flux is more or less constant with wavelength, except in the blue where it shows a rapid decrease short ward 0.55 µm. This is partly due to the lower throughput in the blue (as shown in figure 1) plus a small contribution by extinction. In the red, the lower throughput is balanced by the linear increase of the number of photon per second with wavelength for a given flux.

## Improvement due to AO

The previously listed performances show little difference between AO and non AO detection: the gain is 0.2-0.3 magnitude in continuum and 30% in line flux. This is due to the relatively large area (9 or 16 pixels) used to recover the object flux.  On the other hand, if we restrict to the central pixel, we have a much larger difference. An example in shown in the following table, with 0.8 magnitude gain and 100% gain in flux between AO and non AO observations.

Deep Field Observations (80x1h) – Unresolved Source – Central pixel only

|  (µm) | Band |  | AB full R |  | AB R/10 |  | Line Flux |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Atm. cond | Poor | Good | Poor | Good | Poor | Good |
| 0.79 | I | Non AO | 23.5 | 24.3 | 24.8 | 25.6 | 14.0 | 6.7 |
|  |  | AO | 24.2 | 25.1 | 25.5 | 26.4 | 7.5 | 3.4 |

## Noise regime

### Wide field mode

The following table presents the worst case of noise variance distribution computed in the case of deep field observation of an unresolved source and in poor seeing conditions (non AO mode).

Typical noise variance distribution in wide field mode

|  (µm) | Band | Object | Sky | Read Noise | Dark Current |
| --- | --- | --- | --- | --- | --- |
| 0.465 | B’ | 1.3% | 45.7% | 44.6% | 8.4% |
| 0.55 | V | 0.9% | 75.2% | 20.1% | 3.8% |
| 0.64 | R | 0.9% | 77.2% | 18.4% | 3.5% |
| 0.79 | I | 0.9% | 76.0% | 19.4% | 3.6% |
| 0.93 | z’ | 1.2% | 53.7% | 37.9% | 7.1% |

It shows that we are in photon noise regime in most of the wavelength range with less than 25% of the total variance due to detector noise. It is only at the two extreme wavelengths that the detector noise fraction reaches half of the total variance. These results are for a conservative value of 4 electron readout detector noise. Dark current is almost negligible.

### Narrow field mode

Typical noise variance distribution in narrow field mode

|  (µm) | Band | Object | Sky | Read Noise | Dark Current |
| --- | --- | --- | --- | --- | --- |
| 0.465 | B’ | 19.4% | 3.5% | 65.0% | 12.2% |
| 0.55 | V | 18.5% | 11.5% | 59.0% | 11.1% |
| 0.64 | R | 18.3% | 12.7% | 58.1% | 10.9% |
| 0.79 | I | 18.4% | 12.0% | 58.6% | 11.0% |
| 0.93 | z’ | 19.2% | 4.7% | 64.0% | 12.0% |

As expected in the narrow field mode, the sky contribution is becoming much smaller in the 25 milliarcsec pixel and we are then in a detector noise regime. An improvement of detector read-out noise will have a major impact in this mode.

## Centering star

Optimal merging of the 80 individual 1 hour exposure into the final deep field datacube will required a star bright enough to allow accurate centering. According to the ETC a star brighter than 22.8 RAB magnitude will reach a S/N of 50 after rebinning at low spectral resolution (R~30)  resolution. Even at this low spectral resolution, the full spectral range is sample with 20 points and will allow a good estimate of the spatial PSF and its variation with wavelength.

## Other models of throughput

In the different throughput models presented in AD1, we have selected the typical curve model. Although we think that this model is a realistic goal, it might be considered as difficult to achieve. To show the importance of throughput, we present in the figure 2 a performance comparison when using the worst and best case models.

Figure 2: Limiting deep-field flux (bottom figure) in 10-19 erg.s-1.cm-2 for 3 different throughput models: typical (solid line), worst case (dashed line) and best case (point-dashed line). The corresponding throughput curves are shown in the top figure.

As it can be seen, there is a significant difference between the worst and typical curve. The limiting flux is increased by 30% from 3 to 4 10-19 erg.s-1.cm-2. Although this is not negligible, this is still in phase with the deep field science case. Limiting magnitude difference between the best and typical throughput curves is especially important in the red, with a decrease by a factor 2 at 0.93 µm. Having such a throughput (mainly due to CCD QE), will be very benefit for the deep field science case and it should be a goal for phase B study.

# Limitations

The ETC does not take into account any possible systematic such as flat field residuals and its results should therefore be considered as optimistic. These 2nd order effects are by definition difficult to model in this simple method. We plan to have a more realistic approach using the Instrument Numerical Model (INM). Results from the end-to-end model will be compared to the ETC results.

One possible concern is the accuracy of sky subtraction which may be difficult to achieve in the absence of a beam switching or nod and shuffle technics. Outside OH lines, the sky brightness is around 410-19 erg.s-1.cm-2.Å-1.pixel-1 and is nearly constant in our spectral range. This is 40 times larger than the faintest object we can detect which translate into a few percent precision of sky subtraction. Our experience with SAURON deep fields (see Science Case in RD4) shows that it is easy to correct a factor 10. The remaining factor 4 improvement should be achievable since, unlike SAURON, MUSE will be specifically designed for deep field projects and optimized for very long integration. However this is seen as a critical point and will be addressed in depth in the next project phases.

The ETC point-like sensitivity is based on a summation over a spatial area corresponding to roughly half the object flux. This is a simplistic method and could be improved using optimal summation scheme taking into account the real spatial distribution of signal and noise. We plan to estimate the corresponding gain using the INM and simulated deep fields.

# Plan for phases B & C

In the next phases of the project we will maintain the ETC up to date with the instrument performances. The detailed design should allow a better estimate of throughput, image quality and detector characteristics, and further AO simulations should improve our knowledge of the expected spatial PSF. At the end of phase B, the IFU prototype will also give empirical data with which to calibrate the ETC.

The ETC will be made available to the science team, either via a web interface or via portable software. To ensure accurate and realistic results, we will extensively compare ETC results with full end-to-end simulations using simulated object datacubes, the instrument numerical model and data reduction and analysis software. Updated ETC results will be reassas and optimize the observation strategy accordingly.

# ANNEX – ETC mathcad sheet

Roland Bacon

version 1.0 11-05-03

Inspired from Simon Morris mathcad and Ian Parry excel ETC

version 2.0 8-06-03   Take sky value (no OH) from Hanushik paper

add variation of ensquared energy with wavelength, use updated version 1.1 of throughput

version 3.0 02-10-03

Refurbish and simplify the presentation

updated version 2.1 of throughput

include also bad seeing conditions for AO performances and add effect of MUSE IQE on all PSFs

version 3.1 09-10-03

Change z = 0.95 µm and B=0.44 µm traditional wavelength to the red and blue limits of MUSE (respectively 0.93 µm and 0.465 µm)

version 3.2 11-11-03

Add computation of accuracy needed in sky subtraction

version 3.3 1-12-03

Updated version 20/11/03 of throughput with VLT adaptative secondary AO system, typical curve. Added computation of surface line emission sensitivity.

1. Units and Constant

Velocity of light in vacuum

Angstroem

microns

Planck's constant (h)

back

2. Define a few useful functions

back

3. Photometric System

3.1 UBVRIz System

Bessel, 1979, PASP 91, 589

Magnitude central wavelengths and zero points from ESO web site

http://www.eso.org/observing/etc/doc/gen/formulaBook/node12.html

Useful reference wavelength for MUSE

Central wavelengths to be used

Note that B and z wavelength are set to the limit of MUSE wavelength range

Function to transform magnitude in flux

back

3.2 AB magnitude system

back

4. Sky brightness

Sky brightness is taken from the Hanuschik paper, it doesnt include the OH lines

Checking

ESO ETC value 22.7

ESO ETC value 21.8

ESO ETC value 20.9

ESO ETC value 19.9

ESO ETC value 18.8

Note the difference in the red is fully explained by the OH suppression. The difference in the blue is probably due to the moon light, a moon aged of 5 days would make the difference.

back

5. Atmospheric extinction

Reference: Paranal extinction - ESO

VIMOS ETC

back

6. Telescope Effective Area

From ESO UVES ETC

Note: Useful surface is only

back

7. MUSE throughput

7.1 MUSE throughput of WF mode

Total throughput of MUSE, excluding atmosphere, version 20/11/03

Typical curve with Adaptive Secondary

back

7.2 MUSE throughput of HR mode

8. MUSE CCD characteristics

Dark Current

Readout noise

Note; This is for Fairchild CCD

back

9. MUSE spatial and spectral configurations

9.1 MUSE wide-field spatial mode

9.2 MUSE high spatial resolution mode

9.3 MUSE Spectral characteristics

back

10. MUSE Spatial PSF

10.1 MUSE spatial PSF in WF mode

10.1.1 Seeing limited, poor seeing conditions

Note that MUSE IQE is now included in ensquared energy

back

10.1.2 Seeing limited, good seeing conditions

Note that MUSE IQE is now included in ensquared energy

back

10.1.3 AO Gen I, poor seeing conditions

Note that MUSE IQE is now included in ensquared energy

back

10.1.4 AO Gen I, good seeing conditions

Note that MUSE IQE is now included in ensquared energy

back

10.2 MUSE spatial PSF in HR mode

10.2.1 AO Gen II, good seeing conditions

Note that MUSE IQE is now included in ensquared energy

10.3 Number of spatial pixels

In the case of unresolved objects and in good seeing conditions we will sum up 3x3 spatial pixels to recover a fraction of the object flux, this correspond to 0.6x0.6 arcsec² in WF mode and 0.075x0.075 arcsec² in HR mode

In the case of unresolved objects and in poor seeing conditions we will sum up 4x4 spatial pixels to recover a fraction of the object flux, this correspond to 0.8x0.8 arcsec² in WF mode and 0.1x0.1 arcsec² in HR mode

Note that this choice is somewhat arbitrary. It is a trade between S/N and spatial resolution. Optimum summation should allow increase of the S/N while keeping the spatial resolution.

11. MUSE spectral PSF

11.1 Shape of spectral PSF

back

The spectral PSF is assumed to be Gaussian with 2*pixels FWHM

Fraction of energy enclosed within n pixels :

Low spectral resolution is obtained after summation of N spectral pixels

back

11.2 Number of spectral pixels

To recover major party of the fllux of an emission line we sum over 3 pixels in the spectral direction

Main ETC Formula

Signal to Noise SN

Object Flux

Integration time t

Where  is the Object Flux  ()

if it is a surface brightness, flux should be in

if it is a continuum source, flux should be in

and  is the Sky Flux ( )

and n is the number of exposures

and t is the integration time in sec of one exposure

and RN is the readout noise in electron per pixel

and DC is the dark current in electron per pixel and per hour

the coefficients  are defined below

The coefficient  transform the object flux in photons per second

Where   is the fraction of total flux enclosed in a spectral bin

and  is the fraction of total flux enclosed in a spatial bin

and  is the size of a spectral bin

and  is the linear size of a spatial bin in arcsec

and  is the wavelength

and  is the airmass

and  is the MUSE+VLT total throughput

and  is the effective collective area of VLT primary mirror

and  is the extinction absorption coefficient at Paranal

Note that when the flux is a flat continuum source (flux per A)

must be set to 1 and  to the size of the spectral bin

and when  the flux is an emission source (flux not per A)

must be set to the flux fraction enclosed in the bin and  to 1

Note that when the flux is a surface brightness source (flux per arcsec²)

must be set to 1 and  to the size of the spectral bin

and when  the flux is a total flux (flux not per arcsec²)

must be set to the flux fraction enclosed in the bin and  to 1

The coefficient  transform the sky flux in photons per second

The coefficient  is the number of summed bin

The coefficient  is the number of summed pixels

back

Noise Statistics

This function give the fraction of noise due to object (line 1), sky (line 2), readout (line 3), dark current (line 4), detector (ie readout + drak current, line 5)

back

13. ETC parameters

Signal to Noise

Exposure time

Number of summed exposures

Air mass of observations

Sky flux is taken outside OH lines

back

14. Limiting surface brightness

Estimation of limiting surface brightness for a continuum source with flat spectra. The computation is done by spectral and spatial pixels.

back

14.1 WF mode

back

back

Computing line emission sensitivity by arcsec

We sum the emission line over 3 pixels

back

14.2 HR mode

Nota that the computation is done for a single 1 hour integration

back

back

15. Limiting flux for an unresolved source

15.1 WF mode

15.1.1 Seeing limited, poor seeing conditions

back

15.1.1.1 Continuum source

In case of lower dispersion we have

back

15.1.1.2 Line emission source

We sum the emission line over 3 pixels

back

back

15.1.2 Seeing limited, good seeing conditions

back

back

15.1.2.1 Continuum source

In case of lower dispersion we have

back

15.1.2.2 Line emission source

back

back

15.1.3 AO Gen I, poor seeing conditions

.1.3.1 Continuum source

back

In case of lower dispersion we have

15.1.3.2 Line emission source

back

back

15.1.4 AO Gen I, good seeing conditions

back

15.1.4.1 Continuum source

back

In case of lower dispersion we have

back

15.1.4.2 Line emission source

back

back

15.2 HR mode

15.2.1 AO Gen II, good seeing conditions

back

15.2.1.1 Continuum source

back

In case of lower dispersion we have

back

15.2.1.2 Line emission source

back

back

16. Accuracy requirements in sky subtraction

We compute the ratio of sky flux (outside OH lines) with the object flux

Thus at most the sky is 40 times the object flux and sky subtraction to a precision of 1% should be OK in all cases.

These technics are not applicable to the deep field given that object and sky locations are unknown and wavelength dependant.